| Radiocommunication Study Groups | Received: [date] Subject: Recommendation ITU-R P.619-6 Document 3M/[xx]-E [date] English only |
| --- | --- |

[Switzerland (Confederation of)]

PROPOSED CORRECTION AND CLARIFICATIONS TO ATTACHMENT F

OF RECOMMENDATION ITU-R P.619-6

Summary

This contribution proposes the correction of a suspected typographical error in equation (60b) of Attachment F to Annex 1 of Recommendation ITU-R P.619-6, “Test for whether precipitation-scatter calculation is necessary”, and identifies four further points in Attachment F that would benefit from clarification: the gain of the interfered-with antenna in equation (67), the gaseous attenuation on paths to and from space stations in equations (61) and (67), the angles to be used for the coefficients of Recommendation ITU-R P.838 in equation (63c), and the conditions of applicability of the test. Proposed modifications are given in the Annex.

# 1	Introduction

Attachment F to Annex 1 of Recommendation ITU-R P.619-6 gives a simple test, based on two cylinders representing the antenna beams, to determine whether a full precipitation-scatter calculation is necessary. The points below were identified while developing an open-source implementation of Recommendation ITU-R P.619 together with implementation-independent validation vectors (see § 4). All numerical examples were obtained with that implementation and verified with an independent reference implementation.

# 2	Discussion

## 2.1	Equation (60b): suspected typographical error

Equation (60b) reads:

L2 = max(L1 cos θ, 2r1 sin θ)   (m)	(60b)

The second argument of the max function represents the length of cylinder 2 traversed by cylinder 1. By the same construction as in equation (60a), where the length of cylinder 1 traversed by cylinder 2 is L1 = 2r2/sin θ, this length is 2r1/sin θ, i.e. a division by sin θ rather than a multiplication.

The printed form also leads to an internal inconsistency in Step 6. For two beams of equal radius (r1 = r2) the two cylinders are interchangeable, and equations (65a) and (65b) must give the same scattered e.i.r.p. This requires L2 = L1, which is satisfied for all θ with the division but not with the multiplication. With the printed form, the volume ratio r22L2/(r12L1) in equation (65b) is smaller than one, so that the scattered e.i.r.p. attributed to the common volume, Peirps, exceeds the total power scattered by the cylinder, Pscat (Table 1). With the division, r22L2 ≥ r12L1 for all θ since r2 ≥ r1.

TABLE 1

Cylinder length L2 and Peirps − Pscat from equation (65b) for equal beam radii (r1 = r2 = r)

| θ (degrees) | L2/r, printed (60b) | L2/r, with 2r1/sin θ | Peirps − Pscat (dB), printed (60b) | Peirps − Pscat (dB), with 2r1/sin θ |
| --- | --- | --- | --- | --- |
| 10 | 11.343 | 11.518 | +0.07 | 0.00 |
| 20 | 5.495 | 5.848 | +0.27 | 0.00 |
| 30 | 3.464 | 4.000 | +0.62 | 0.00 |
| 45 | 2.000 | 2.828 | +1.51 | 0.00 |
| 60 | 1.732 | 2.309 | +1.25 | 0.00 |
| 75 | 1.932 | 2.071 | +0.30 | 0.00 |
| 90 | 2.000 | 2.000 | 0.00 | 0.00 |

As a consequence, the result of the test for equal beams depends on which beam is designated as cylinder 1. For example, at 14.25 GHz with r1 = r2 = 200 m, θ = 45° and Rrain = 50 mm/h, the scattered e.i.r.p. is 24.01 dB(W) when the unwanted beam is cylinder 1 and 24.24 dB(W) when it is cylinder 2; with the division, both give 24.01 dB(W).

The two forms coincide at θ = 90°. Moreover, whenever r2 cos θ > r1 (for example r2 = 2r1 and θ < 60°) the term L1 cos θ dominates and both forms give the same result, which may explain why the error has not been noticed.

## 2.2	Equation (67): gain of the interfered-with antenna

Step 8 estimates “the unwanted scattered power received by the interfered-with antenna”, which is then compared with the receiver’s interference threshold Pint. Equation (67) contains the scattered e.i.r.p., the factor Fnis, the free-space basic transmission loss 92.4 + 20 log(drx f) and the gaseous attenuation, but no gain of the interfered-with antenna; Ptxs is therefore the power that would be received by an isotropic antenna. The gain of the unwanted transmitter, on the other hand, is included explicitly in equation (61a).

If Pint is a threshold at the receiver input, the test underestimates the interfering power by the gain of the interfered-with antenna in the direction of the common volume. Since the common volume lies within the main beam of that antenna, this is typically several tens of dB. For example, with Ptxs = −113.3 dB(W) and Pint = −88.3 dB(W), Pint − Ptxs = 25 dB and the test concludes that no full calculation is needed, whereas with a receive antenna gain of 40 dBi the power at the receiver input is −73.3 dB(W), i.e. 15 dB above the threshold.

WP 3M is invited either (a) to include the gain Grx of the interfered-with antenna in the direction of the common volume in equation (67), as proposed in the Annex, or (b) to state explicitly that Pint is to be referred to an isotropic antenna.

## 2.3	Equations (61) and (67): gaseous attenuation on paths involving a space station

Equations (61) and (67) account for gaseous attenuation as γg dtx and γg drx, i.e. a specific attenuation multiplied by the path length. This is appropriate for terrestrial paths but not when the unwanted transmitter or the interfered-with receiver is a space station, which is within the scope of this Recommendation. For example, at 20 GHz, for a common volume at a height of 2 km and a geostationary satellite seen at an elevation of 30.8° (dtx ≈ 38 540 km), γg dtx is about 4 190 dB with the specific attenuation at sea level (0.109 dB/km), and still about 1 700 dB with the specific attenuation at 2 km (0.044 dB/km), whereas the gaseous attenuation along the slant path calculated with Attachment C is 0.21 dB.

It is proposed to replace γg dtx and γg drx by the gaseous attenuation along the respective path, Ag,tx and Ag,rx, equal to γg d for terrestrial stations and calculated with Attachment C between the height of the common volume and the space station otherwise.

## 2.4	Equation (63c): angles for the coefficients of Recommendation ITU-R P.838

Equation (63c) gives the specific rain attenuation γr = k Rrainα, with k and α from Recommendation ITU-R P.838. These coefficients depend on the path elevation angle and on the polarization tilt angle, which are not specified in Attachment F. Since γr is used in Step 4 to attenuate the power travelling along the cylinder representing the unwanted beam, it is suggested to use the elevation angle of the axis of the unwanted beam and the polarization tilt angle of the unwanted transmission. The choice is significant: at 14.25 GHz and Rrain = 50 mm/h, γr is 3.33 dB/km for horizontal and 2.74 dB/km for vertical polarization on a horizontal path, and 3.00 dB/km for either polarization at an elevation of 90°, i.e. a difference of about 20%.

## 2.5	Conditions of applicability

Attachment F states that “this test should not be used if the horizontal extent of the common volume exceeds 6 600 Rrain−0.08 metres” and that “the test is considered reliable for 10 ≤ θ ≤ 90 degrees”. The horizontal extent of the common volume is not defined, and neither statement says how to proceed when the condition is not met. It is suggested to define the horizontal extent of the common volume (for instance in terms of the cylinder lengths and the elevation angles of the beam axes) and to state that a full precipitation-scatter calculation should be conducted when the test is not applicable.

# 3	Proposal

WP 3M is invited to:

- correct equation (60b) as described in § 2.1;

- consider the clarifications described in §§ 2.2 to 2.5; and

- revise Attachment F to Annex 1 of Recommendation ITU-R P.619-6 accordingly, taking into account the proposed modifications in the Annex to this contribution.

# 4	Implementation and validation

An open-source MATLAB/Octave implementation of Recommendation ITU-R P.619, together with implementation-independent validation vectors and an independent reference implementation, is available at https://github.com/eeveetza/p619. The implementation and the validation vectors for Attachment F use equation (60b) with the division and include pairs of equal-beam cases that test the consistency described in § 2.1. They can be used to check other implementations of the Recommendation.

| Contact: | Ivica Stevanovic, Federal Office of Communications (OFCOM), Switzerland E-mail: [e-mail address] |
| --- | --- |

ANNEX

Proposed modifications to Attachment F to Annex 1 of Recommendation ITU-R P.619-6

Modifications are shown as tracked changes. Unchanged parts of the text are indicated by “*   *   *”.

Attachment F

Test for whether precipitation-scatter calculation is necessary

*   *   *

This test should not be used if the horizontal extent of the common volume exceeds 6 600 Rrain−0.08 metres, where the horizontal extent of the common volume is [to be defined by WP 3M]. If this condition is not met, or if θ is outside the range given below, the test is not applicable and a full precipitation-scatter calculation should be conducted.

The test is considered reliable for 10 ≤ θ ≤ 90 degrees.

*   *   *

Step 1: Calculate the cylinder lengths:

L1 = 2r2/sin θ   (m)	(60a)

L2 = max(L1 cos θ, 2r1 /sin θ)   (m)	(60b)

Step 2: Calculate the power-flux density S in dB(W) incident upon the circular end of the cylinder representing the unwanted beam:

S = Peirp − 20 log(dtx) − Ag,tx − 71.0   dB(W/m2)	(61)

where:

Peirp:	e.i.r.p. of the unwanted transmitter in dB(W)

dtx:	distance from the unwanted transmitter to the common volume in km

γg:	specific attenuation due to atmospheric gasses in dB/km

Ag,tx:	attenuation due to atmospheric gases in dB along the path between the unwanted transmitter and the common volume. For a terrestrial transmitter, Ag,tx = γg dtx. For a space station, Ag,tx is calculated using Attachment C between the height of the common volume and the space station.

*   *   *

Step 4: Calculate the power leaving the other end of the cylinder representing the unwanted beam:

*   *   *

γr = k Rrainα   dB/km	(63c)

where:

Rrain:	point rainfall rate in mm/h for a 1 minute integration time exceeded for p% time

k and α:	regression coefficients given by Recommendation ITU-R P.838, evaluated for the elevation angle of the axis of the unwanted beam and the polarization tilt angle of the unwanted transmission.

*   *   *

Step 8: Estimate the unwanted scattered power received by the interfered-with antenna using:

Ptxs = Peirps + Fnis + Grx − 20 log(drx f) − Ag,rx − 92.4   dB(W)	(67)

where:

drx:	distance in km of the interfered-with antenna from the common volume represented by the cylinders

Grx:	gain in dBi of the interfered-with antenna in the direction of the common volume

Ag,rx:	attenuation due to atmospheric gases in dB along the path between the common volume and the interfered-with antenna, calculated as Ag,tx in Step 2 with drx in place of dtx.

A full rain-scatter calculation should be conducted if Pint − Ptxs < 20 dB, where Pint is the receiver’s interference threshold in dB(W) at the receiver input.

*   *   *