| Radiocommunication Study Groups |  |
| --- | --- |
| Received: [date] Subject: Recommendation ITU-R P.619 | Document 3J/[xx]-E Document 3L/[xx]-E Document 3M/[xx]-E |
|  | [date] |
|  | English only |
| Switzerland (Confederation of) |  |
| IDENTIFIED INCONSISTENCIES AND CLARIFICATIONS REQUIRED IN RECOMMENDATION ITU-R P.619-6 |  |

# 1	Introduction

Document 3M/295 identified a number of ambiguities, inconsistencies and missing definitions encountered during a software implementation of Recommendation ITU-R P.619-5. Recommendation ITU-R P.619-6 has since been published. This contribution reviews each of the issues of Document 3M/295 against the text of Recommendation ITU-R P.619-6 and reports further issues found while the implementation (https://github.com/eeveetza/p619) was extended.

The implementation now covers the basic transmission loss for single-entry (Section 3.1, equation (14)) and multiple-entry interference (Section 3.2, equation (15)), the propagation mechanisms of Section 2, including diffraction/ducting (Section 2.6) and clutter and building entry loss (Section 2.7), and Attachments A to F. It is accompanied by an implementation-independent validation suite (2 367 test values in 27 files, with an independent reference implementation), which is checked against the ITU-R Study Group 3 validation examples where these exist.

For each issue, the status in Recommendation ITU-R P.619-6 is given, the approach adopted in the implementation is described, and a proposed action is stated. Issues specific to Attachment F (test for the need of a precipitation-scatter calculation) are addressed in a separate contribution.

# 2	Issues carried over from Document 3M/295

## 2.1	Attenuation due to atmospheric gases: time-percentage dependence of Ag(p1)

Equation (14) of Recommendation ITU-R P.619-6 retains the time-percentage dependence of the attenuation due to atmospheric gases:

Lb=Lbfs+Axp+Agp1+Abs+Asp2+Ldtb(p1)

Status in P.619-6: Attachment C itself still contains no time-percentage parameter. Section 2.3 refers to Recommendation ITU-R P.836 for the statistics of surface water-vapour density and to the scaling of the water-vapour density with height, ρ(h) = ρ0 exp(−h/2), and Section 4 indicates that ρ0 is an input to Attachment C. This makes it possible to introduce a time-percentage dependence, but the procedure that links p1 to ρ0 is not stated. In particular, it is not stated:

a)	that Ag(p1), an attenuation not exceeded for p1 % of the time, requires the water-vapour density exceeded for q = 100 − p1 % of the time;

b)	how to proceed for p1 < 1 %, since the Recommendation ITU-R P.836 maps cover 0.1 % ≤ q ≤ 99 % only, whereas small values of p1 are typical for interference studies;

c)	at which location the water-vapour density is to be taken (earth-based station, or another point of the path);

d)	how the surface value ρs at the altitude of the station is converted to the sea-level value ρ0 of the exponential profile;

e)	whether only the water-vapour profile varies with p1 (temperature and dry pressure remaining those of the reference atmosphere), and whether the dependence applies only to the mean annual global reference atmosphere of Recommendation ITU-R P.835 or also to the other reference atmospheres.

Implementation: As an option (the default remains the reference value ρ0 = 7.5 g/m3), the implementation computes ρs exceeded for q = 100 − p1 % from the Recommendation ITU-R P.836-6 annual maps at the location and altitude He of the earth-based station, sets ρ0 = ρs exp(He/2) and uses ρ(h) = ρ0 exp(−h/2) in the mean annual global reference atmosphere, with the mixing-ratio floor of Recommendation ITU-R P.835. For q outside 0.1–99 %, the nearest limit is used and a warning is issued. Temperature and dry pressure are not changed. The surface water-vapour density part reproduces the 32 ITU-R validation examples of Recommendation ITU-R P.836-6 to within 10−13 g/m3.

Table 1 shows the effect for an earth station in Bern (46.95° N, 7.44° E, He = 0.55 km) and a geostationary space station at 0° E (elevation angle ≈ 35.5°), at 20 GHz. With the reference value ρ0 = 7.5 g/m3, Ag = 0.34 dB for any p1.

TABLE 1

Example of Ag(p1) with Recommendation ITU-R P.836-6 (20 GHz, Bern to GSO at 0° E)

| p1 (%) | q (%) | ρs (g/m3) | ρ0 (g/m3) | Ag (dB) |
| --- | --- | --- | --- | --- |
| ≤ 1 | 99 | 2.84 | 3.74 | 0.215 |
| 10 | 90 | 4.19 | 5.52 | 0.274 |
| 50 | 50 | 7.39 | 9.72 | 0.415 |
| 90 | 10 | 12.15 | 16.00 | 0.630 |
| 99 | 1 | 14.60 | 19.22 | 0.742 |

Proposed action: Specify in Section 2.3 or in Attachment C the procedure by which ρ0 is derived from p1, covering items a) to e) above, or confirm the approach adopted in the implementation.

## 2.2	Upper height limit for atmospheric integration (Attachment C)

Status in P.619-6: Unchanged. No upper altitude limit is defined for the integration of Attachment C, whereas Recommendation ITU-R P.676 and the reference atmospheres of Recommendation ITU-R P.835 are defined up to 100 km.

Implementation: Document 3M/295 reported that Hs = min(100, Hs) km had been applied. This has been revised, because Hs is also used in the geometry of Attachment A (path length, elevation angle) and in the free-space loss, and limiting it there gives wrong results for space stations above 100 km. The implementation now limits only the atmospheric layers: the layers follow equation (16) of Recommendation ITU-R P.676 up to min(Hs, 100 km), and above 100 km the refractive index is taken as 1 and the gaseous attenuation as zero. Hs is not modified elsewhere.

Proposed action: State in Attachment C that the integration extends up to min(Hs, 100 km), that the atmosphere above 100 km is treated as free space (n = 1), and that Hs is not limited in the other parts of the Recommendation.

## 2.3	Definition of h in Recommendation ITU-R P.834, Section 5 (beam spreading loss, Section 2.4.2)

Status in P.619-6: Unchanged. Section 2.4.2 refers to Section 5 of Recommendation ITU-R P.834, where h is "the altitude of the lower point above sea level (km)", with h ≤ 5 km. It is also not stated what value Abs takes outside the validity range of the method (θ0 ≤ 10°, h ≤ 5 km).

Implementation: h = He, the altitude of the earth-based station. Abs is taken as a positive loss and is set to 0 outside the validity range (the beam spreading loss is negligible there).

Proposed action: Clarify in Section 2.4.2 that h is the altitude of the earth-based station and that Abs = 0 outside the validity range of the method.

## 2.4	Ionospheric scintillation (Section 2.5.1): non-distributable software dependency

Status in P.619-6: Section 2.5.1 states that ionospheric scintillation can be ignored above 10 GHz. The method continues to rely on Recommendation ITU-R P.531, whose integral software is written in Fortran under a licence that restricts redistribution and limits use to scientific purposes. An openly distributable method is still not available.

Implementation: Ionospheric scintillation is not implemented. This is documented as a limitation for frequencies below 10 GHz.

Proposed action: Consider whether an openly distributable method for ionospheric scintillation could be referenced, or define the conditions under which this term may be neglected below 10 GHz.

## 2.5	Attachment C.5: virtual terminal and path geometry

The ambiguities of Attachment C.5 reported in Document 3M/295 remain in Recommendation ITU-R P.619-6. A further case (Section 2.5.4) has been identified.

### 2.5.1	Beamwidth of the virtual terminal

Status in P.619-6: Unchanged; the beamwidth of the virtual terminal is not defined.

Implementation: The virtual terminal is treated as having no antenna: the beamwidth check is not applied to the path between the earth-based station and the virtual terminal. The check of the space-station beamwidth is kept for the second path. This is equivalent to the 180° beamwidth assumed in Document 3M/295.

Proposed action: State that no beamwidth check applies to the virtual terminal.

### 2.5.2	Path geometry divergence between Recommendations ITU-R P.619 and ITU-R P.676

Status in P.619-6: Unchanged. Attachment C.5 defines two paths for use in equation (44) that differ from the definitions of Recommendation ITU-R P.676:

|  | Rec. ITU-R P.676 definition | Rec. ITU-R P.619 definition |
| --- | --- | --- |
| Path 1 | Gaseous attenuation between a virtual earth station at height hG and the actual earth station at height h1, at an apparent elevation angle of 0° (descending). | Extends from the Earth-based transmitter to the virtual terminal (ascending) |
| Path 2 | Gaseous attenuation between a virtual earth station at height hG and the maximum atmospheric height (typically 100 km), at an apparent elevation angle of 0° (ascending) | Extends from the virtual terminal to the space-based receiver (ascending) |

Implementation: Both paths are evaluated as ascending rays launched horizontally (apparent elevation angle 0°) from the virtual terminal at Hmin, one up to He and one up to Hs. By reciprocity, the first path gives the attenuation of the descending path from the earth-based station to the virtual terminal; this is the approach of Recommendation ITU-R P.676. Hmin is obtained from equation (45) with the iteration of equations (46) and (47).

Proposed action: Harmonise the path definitions in Attachment C.5 with those of Recommendation ITU-R P.676.

### 2.5.3	Atmospheric profile recalculation

Status in P.619-6: Unchanged. It is not specified whether the profiles ρ, T, P and n are to be recalculated for the paths from Hmin to He and from Hmin to Hs.

Implementation: For each path, new layers are formed with equation (16) of Recommendation ITU-R P.676 starting at Hmin, and the profiles are evaluated at the mid-point of each layer.

Proposed action: Specify that the layering and the profiles are recalculated for each of the two paths, starting at Hmin.

### 2.5.4	Ray intersecting the Earth (Hmin < 0)

For a sufficiently negative elevation angle φe at a low earth station, the solution of equation (45) gives Hmin < 0, i.e. the ray would pass below sea level. This case is not addressed.

Implementation: No Earth-space path exists; Ag is set to 1020 dB (the value used by Attachment C for rays missing the antenna beam) and a warning is issued.

Proposed action: State how the case Hmin < 0 is to be handled.

## 2.6	Wet term of refractivity: enhancement factor in Recommendation ITU-R P.453-14

In applying the formula for the wet term of refractivity from Recommendation ITU-R P.453-14 (Annex 1, equation (9)), an ambiguity arises regarding the treatment of the enhancement factor EF at temperatures below 0 °C:

(9)

and

where

t: 	temperature (oC)

P: 	total atmospheric pressure (hPa)

H:	relative humidity (%)

es:	saturation vapour pressure (hPa) at the temperature t (C) and the coefficients a, b, c and d are:

for water	for ice

a    6.1121	a    6.1115

b    18.678	b    23.036

c    257.14	c    279.82

d  =  234.5	d  =  333.7

(valid between −40 to 50	(valid between −80 to 0

It is not clear from the text whether, for temperatures between −40 °C and 0 °C, the enhancement factor should be EF = EFwater or EF = EFice (or a combination thereof).

Status in P.619-6: Unchanged (the issue lies in Recommendation ITU-R P.453-14).

Implementation: EF = EFwater is assumed throughout. Note that in the calculation of tropospheric scintillation, Nwet is now taken from the digital maps of Recommendation ITU-R P.453, so that equation (9) is used only when Nwet is computed from local meteorological data.

Proposed action: Clarify in Recommendation ITU-R P.453-14, or in a note in Recommendation ITU-R P.619, how the enhancement factor EF should be computed for temperatures below 0 °C.

# 3	Further issues identified in Recommendation ITU-R P.619-6

## 3.1	Attachment A: argument order of atan2 in equations (23) and (24)

Attachment A writes the elevation angle as θ0 = atan2(Gts, Z2) and defines atan2(x, y) = arctan(x/y). Applied literally, equation (23) gives the zenith angle instead of the elevation angle, and equation (24) gives an incorrect bearing. The equations are consistent with the definitions of elevation and bearing only if the arguments are read in the opposite order, i.e. θ0 = arctan(Z2/Gts) and ψ = arctan(Y2/X2), with the quadrant resolved from the signs of both arguments.

Implementation: θ0 = arctan(Z2/Gts) and ψ = arctan(Y2/X2) (four-quadrant).

Proposed action: Correct either the definition of atan2 or the order of the arguments in equations (23) and (24).

## 3.2	Section 2.6: diffraction parameter in the worked example

The worked example in Section 2.6 (Figure 10) quotes ν ≈ 4.9 for h = 60.3 m, d1 = 24 km and 30 GHz. Equation (12a) gives ν = 5.51 for these values; the quoted value results from using the Fresnel radius at 30 km instead of 24 km.

Proposed action: Correct the value of ν in the worked example (and the resulting loss, if applicable).

## 3.3	Section 2.6: reference to the slant-path method of Recommendation ITU-R P.526

Section 2.6 refers to Section 4.6 of Recommendation ITU-R P.526 for terrain diffraction on slant paths. This section exists only in Recommendation ITU-R P.526-16; the reference is undated. It is also not stated explicitly that the ray traced with Attachment E is to be used as the direct ray of Recommendation ITU-R P.526, nor how heights measured vertically relative to the ray are to be converted to the distances along, and heights perpendicular to, the ray that the slant-path method uses.

Implementation: The slant-path method of Recommendation ITU-R P.526-16, Section 4.6 (Bullington construction, effective plane-Earth loss and blending) is the default; the traced ray is the direct ray, and vertical heights Δh at horizontal distance d are converted to s = d/cos θ + Δh sin θ and n = Δh cos θ. A single knife-edge approximation, using equations (12a) and (12b) at the profile point with the largest ν, remains available. Equation (13) is applied to terrain obstruction only.

Proposed action: Reference Recommendation ITU-R P.526-16 (or later) explicitly and describe how the traced ray and the terrain profile are used with the slant-path method.

## 3.4	Section 2.7: clutter loss and building entry loss

Section 2.7 refers to Recommendation ITU-R P.2108-0 for clutter loss. Recommendation ITU-R P.2108-1 is in force, and the draft revision P.2108-2 extends the Earth-space clutter model to 0.5 GHz and introduces the station height and the mean clutter height as inputs. Furthermore, it is not specified whether the free-space elevation angle θ0 or the apparent elevation angle θ is to be used for Lc and Lbe in equation (15).

Implementation: The clutter model of draft Recommendation ITU-R P.2108-2, Section 3.3 is the default (Recommendation ITU-R P.2108-1 remains selectable). Lc and Lbe (Recommendation ITU-R P.2109) use the apparent elevation angle θ.

Proposed action: Update the reference to Recommendation ITU-R P.2108 and specify which elevation angle is used for Lc and Lbe.

## 3.5	Attachment D: wet term of the surface refractivity Nwet

Attachment D calculates the tropospheric scintillation with the method of Section 2.4.1 of Recommendation ITU-R P.618-12, which has been superseded (Recommendation ITU-R P.618-14 is in force). Attachment D does not state how Nwet is to be obtained, nor for which percentage of time. Section 4 indicates that Section 1 of Recommendation ITU-R P.453-14 gives Nwet as a function of the water-vapour partial pressure e, which suggests a calculation from local meteorological data, whereas Recommendation ITU-R P.618-14 and the ITU-R Study Group 3 validation examples use the median value of Nwet from the digital maps of Recommendation ITU-R P.453-14 at the site of the earth station.

As a result, implementations may differ in the value of Nwet, for example by using the map value exceeded for p2 % of the time instead of the median, or by deriving Nwet from the water-vapour density used for Ag(p1) (see Issue 2.1), which Section 4 relates to the correlation between gaseous attenuation and scintillation. An earlier version of the implementation used the value exceeded for p2 %; for Bern (46.95° N, 7.44° E) and p2 = 1 %, the value exceeded for 1 % of the time is Nwet = 81.6 instead of the median 40.2, and the scintillation enhancement at 20 GHz (elevation 35.5°, Ga = 40 dBi) becomes 0.34 dB instead of 0.22 dB.

Implementation: Nwet is the median (50 %) value of the Recommendation ITU-R P.453-14 annual maps at the earth-based station (bilinear interpolation, Recommendation ITU-R P.1144), independently of p2; the percentage p2 applies only to the scintillation fade or enhancement. A value of Nwet computed from local data can be given instead. The map interpolation reproduces the 8 ITU-R validation examples of Recommendation ITU-R P.453-14 to within 10−12.

Proposed action: Update the reference in Attachment D to Recommendation ITU-R P.618 (current version), state that Nwet is the median value at the earth-based station from the digital maps of Recommendation ITU-R P.453 (or from local data, where available), that p2 applies only to the scintillation, and align Section 4 accordingly.

# 4	Summary of identified issues

TABLE 2

Summary of issues, status in Recommendation ITU-R P.619-6 and approach adopted in the implementation

| # | Location | Issue | Status in P.619-6 | Implementation |
| --- | --- | --- | --- | --- |
| 2.1 | § 2.3, § 4, Att. C | Procedure linking p1 to ρ0 for Ag(p1) not stated | Partly addressed (P.836 referenced) | q = 100 − p1, P.836-6 at earth station, ρ0 = ρs exp(He/2) |
| 2.2 | Att. C | No upper height limit for the integration | Open | Layers up to min(Hs, 100 km), n = 1 above; Hs not limited |
| 2.3 | § 2.4.2 | h in P.834 § 5 undefined; Abs outside validity range | Open | h = He; Abs = 0 outside range |
| 2.4 | § 2.5.1 | P.531 software not distributable | Negligible above 10 GHz stated | Not implemented |
| 2.5.1 | Att. C.5 | Beamwidth of virtual terminal undefined | Open | No beamwidth check |
| 2.5.2 | Att. C.5 | Path definitions differ from P.676 | Open | P.676 approach (ascending from Hmin) |
| 2.5.3 | Att. C.5 | Profile recalculation not specified | Open | Recalculated per path |
| 2.5.4 | Att. C.5 | Case Hmin < 0 not addressed | New | Ag = 1020 dB, warning |
| 2.6 | P.453-14 | EF below 0 °C unclear | Open | EF = EFwater |
| 3.1 | Att. A | atan2 argument order in eqs (23), (24) | New | arctan(Z2/Gts), arctan(Y2/X2) |
| 3.2 | § 2.6 | Worked example: ν ≈ 4.9 instead of 5.51 | New | Eq. (12a) |
| 3.3 | § 2.6 | Undated reference to P.526 § 4.6; use of traced ray | New | P.526-16 § 4.6; knife-edge option |
| 3.4 | § 2.7, § 3.2 | P.2108-0 reference; elevation angle for Lc, Lbe | New | Draft P.2108-2; apparent elevation |
| 3.5 | Att. D, § 4 | Source and time percentage of Nwet not stated; reference to P.618-12 | New | Median of P.453-14 maps at earth station; Nwet may be given |

# 5	Proposals

Working Party 3M is invited to:

1	review the issues identified in this contribution and determine whether editorial or technical corrections are needed in Recommendation ITU-R P.619-6;

2	specify the procedure for the time-percentage dependence of the attenuation due to atmospheric gases (Issue 2.1);

3	include explicit limits and definitions where these are currently absent (Issues 2.2, 2.3, 2.5.1, 2.5.3 and 2.5.4);

4	consider whether an openly distributable ionospheric scintillation method should be referenced, or whether criteria for neglecting this term should be defined (Issue 2.4);

5	harmonise the path geometry definitions of Attachment C.5 with Recommendation ITU-R P.676 (Issue 2.5.2);

6	correct equations (23) and (24) of Attachment A and the worked example of Section 2.6 (Issues 3.1 and 3.2);

7	update the references to Recommendations ITU-R P.526 and ITU-R P.2108 and clarify their use (Issues 3.3 and 3.4);

8	specify the source and time percentage of Nwet in Attachment D and update the reference to Recommendation ITU-R P.618 (Issue 3.5);

9	invite Working Party 3J to clarify the treatment of the enhancement factor in Recommendation ITU-R P.453-14 at temperatures below 0 °C (Issue 2.6).

______________

____________________